SHOOTING WITH THE MOMENT PRO CAMERA

Moment Field Guide

Version 3, updated 28 August 2026

For Marnie Crawford Samuelson. Compiled from working sessions, May through August 2026.

How to use this guide Part 1 has the checklists you run before shooting, and Part 4 has the end of day workflow. Those two parts carry most of the weight. Everything else is reference for when you want it, and none of it needs to be memorized. That is the whole point of writing it down. This version folds in the 28 August session. Anything that changed carries an UPDATED 28 AUG tag on its heading, and the table just below collects the changes in one place. Inside a tagged section, a small amber dot marks the exact lines that are new, so nothing is buried.

New in Version 3

| What changed | Where it stands now |
| --- | --- |
| Auto Connect is now off | This solved the mystery of shots landing on the SD card when you had chosen the photo library. Sliding the phone into the grip was switching the storage location on its own. Nothing you did wrong. |
| ISO starts at 400 | Saved into both Natural and ProRAW. Your reasoning, and it is the right one: easier to bring ISO down than to discover a shutter too slow to hold, especially with the ND on. |
| File Name Format is Default, not Date | Tested side by side on both phones. Default gives the clean IMG numbers. Video ignores the setting entirely and always uses the long timecode name. |
| A copyright step joins the workflow | Photo Mechanic writes your IPTC copyright into the exported files, then a Finder rename tags them. This is now the last thing that happens before anything goes out. Part 4 has the sequence. |
| Autofocus: let the camera choose | You tried it at Hyannis and everything came back sharp. The lens is wide with deep depth of field, so tapping the white box mostly costs you the moment. |
| Your drive setup checks out | Walked through all three together. Sessions Primary, Sessions Backup, and the Raw Captures Archive are all set up correctly. |

CONTENTS

Part 1.  Before you shoot

The photo checklist

The video checklist

Before you leave the house

Part 2.  Your profiles and critical settings

Which profile, and when

What is in each profile

Changing a setting and saving it

Where your shots are being saved

Part 3.  In the field

Exposure: ISO priority for photo

Focus

Reading the histogram, waveform, and zebras

The variable ND filter

Contrasty light: ProRAW first

Blocking interruptions

When something goes wrong

Part 4.  After the shoot: the media workflow

The shape of it

Before the shoot: set up the session

End of day, step by step

Copyright, before anything goes out

Your drives

The archive, separately

Part 5.  Reference: set once, then forget

Moment settings

iPhone settings

Why these formats, in plain terms

Open questions and next sessions

# Part 1.  Before you shoot

Two checklists, one for photo and one for video. Run the one that matches. After a few outings this takes under a minute, and it means nothing has to be decided in the middle of a scene.

## The photo checklist   UPDATED 28 AUG

|  | Do Not Disturb is on Swipe down from the top right corner, tap Focus. A moon appears by the clock. It turns off the same way when you are done. |
| --- | --- |
|  | The right profile is loaded Natural for everyday. ProRAW for contrasty light or anything you might print. The text along the top bar confirms which one you are in. |
|  | Storage says Photo Library, not SD card ● Look at the small icon on the shooting screen. If it shows the card symbol instead of your last image, the location has switched. With Auto Connect now off this should stay put, but it is worth a glance. |
|  | You are in ISO mode, starting around ISO 400 ● Pick the ISO, let the app find the shutter. The shutter reading sits just above the shutter button, labelled SS. Bring the ISO down if the light allows. |
|  | The ND is mounted if you are outdoors Dial it until the zebras clear. No need to go anywhere near its darkest extreme. |
|  | EV where you want it Zero to start. On Natural, riding at -0.3 to -0.7 protects the highlights, and the raw file holds the shadows. |
|  | White balance is set Daylight outside. Auto for genuinely mixed light. |
|  | Autofocus is on, and you can leave it alone ● The lens is wide and the depth of field is deep. Tapping the white box is optional, and mostly worth skipping so you keep both hands on the grip. |
|  | Exposure looks right, no clipping The things to watch: a spike against the right edge of the histogram, or a solid white line across the top of the waveform. |
|  | Quick Actions are swept Hand icon. Level on, Monitors on, Clipping on at Highlights 99%, Flash off, Timer off, Grid to taste. |
|  | Lenses are clean The edges where glass meets housing collect the most. |
|  | Bluetooth shutter connected, if you are using it |

Why the checklist earns its keep The app hides a lot behind long presses and nested menus, and mid-scene there is no time to go hunting. Everything on this list can be settled while you are still standing comfortably at the location. Then the shooting is just shooting.

## The video checklist

|  | Do Not Disturb is on |
| --- | --- |
|  | The 4K profile is loaded H.264, 4K, SDR, 24 fps. |
|  | Shutter is 1/48 Double the frame rate, the 180 degree rule. This is what gives motion its natural look, so video stays in shutter mode even though photo has moved to ISO mode. |
|  | ND is on and dialed in At 1/48 in daylight the ND is doing the exposure work. Rotate until the zebras clear, and keep clear of the darkest extreme. |
|  | Storage says what you expect ● Same glance as the photo list. Video is usually happier on the SD card given the file sizes, but make it a choice rather than a surprise. |
|  | Stabilization is on |
|  | Audio source is set Quick Actions, Monitors on, tap the audio meters. Source: Built-in, then Back microphone, Stereo. |
|  | White balance is set Daylight outside. Lock Auto White Balance on Record is already on, so Auto will not drift mid-clip. |
|  | Exposure checked, no clipping |
|  | Red record border shows when you roll Your confirmation that it is really recording. |
|  | Lenses are clean |

## Before you leave the house

The night before, or the morning of.

### On the Mac

- Create the Capture One session for the shoot on the primary drive, with its subfolders named and ready. Part 4 walks through this. Having it waiting means the end of the day is drag and drop instead of decisions.

### The phone and the rig

- Batteries charged and on the cage.

- SD card cleared: files deleted on the Mac and the Trash emptied, since deleting alone does not free the space. Card back in the cage.

- To check iPhone storage space: Settings -> General -> About… scroll down to Capacity and Available.

- Lenses cleaned.

- Variable ND on if the day calls for it.

- Room on the phone if you are shooting to the Photo Library.

### The bag

- Reading glasses, lens cleaner and microfiber, the small wrench and screwdriver for the cage, water, notebook and pen.

- Zoom recorder if you are capturing separate audio.

# Part 2.  Your profiles and critical settings

The ten second orientation Settings live in four places, and knowing which one you are in makes the app much friendlier. 1. The gear icon, lower left. App-wide preferences, set once and rechecked after updates. 2. The profile tab. Saved recipes, switched when you change what you are shooting. 3. The hardware icon, the little hexagon. Storage location and external devices. 4. The shooting screen and Quick Actions. Exposure, white balance, focus, clipping. The scene by scene controls.

## Which profile, and when

| Profile | Reach for it when | Good to know |
| --- | --- | --- |
| Natural | Your default for everyday shooting. A shareable JPEG plus a raw file, saved together. | The most film-like result the phone gives, but the least highlight headroom, so it clips first. Riding EV a little low covers this. Locked to the physical lens, 12 MP. |
| ProRAW | Contrasty light, indoor or out, and anything that might get printed. | Fusion stacks exposures, so it holds highlights noticeably better. It is HDR, which looks right on your phone but can read differently on other screens, so plan a conversion step when handing files off. |
| 4K Video | Video. | H.264 is SDR with less grading latitude, and in exchange it edits in Premiere without proxies. |

Low light, for now A dedicated low light profile is still on the list. Until then: ProRAW on the 1x lens is the stronger choice in dim light, since Fusion stacks exposures in a way Natural cannot.

## What is in each profile   UPDATED 28 AUG

Across all three: auto-save is off, and clipping is on at Highlights 99%.

### Natural

| Setting | Value |
| --- | --- |
| Format | Natural, JPEG |
| Keep RAW | On. This is the important one, a raw DNG alongside every shot. (Under Jpeg at the top) |
| Dynamic range | Light, or Off. Strong pushes the image further into computational processing than you want here. |
| Colour space | SDR |
| Resolution | 12 MP, 4:3 |
| ISO | 400 as the starting point ● |
| ISO limits | Minimum None, Maximum 1600 (This is a Shutter priority setting). |
| Storage | Photo Library ● |
| Clipping | Highlights, 99% |

- The JPEG and the raw save together as one item. In the Moment preview, the small red badge at the bottom flips between them.

### ProRAW

| Setting | Value |
| --- | --- |
| Format | ProRAW |
| Fusion | Allowed. Disabling it drops resolution to 12 MP, so it stays on. |
| Colour space | HDR. 48 MP requires it, and the resolution is the reason to be here. |
| Resolution | 48 MP. Treat it as a request; the hardware sometimes delivers around 20 MP, and the stock camera behaves the same way. |
| JPEG companion | Included. Costs little, handy for a quick send. |
| Aspect ratio | 4:3 |
| ISO | 400 as the starting point ● |
| ISO limits | Minimum None, Maximum 1600 (This is a Shutter priority setting). |
| Storage | Photo Library ● |
| Clipping | Highlights, 99% |

### 4K Video

| Setting | Value |
| --- | --- |
| Codec | H.264, Bitrate High |
| Resolution | 4K, SDR |
| Frame rate | 24 fps |
| Shutter | 1/48 |
| Audio | Built-in, Back microphone, Stereo |
| Clipping | Highlights, 99% |

## Changing a setting and saving it   UPDATED 28 AUG

Two different things live under the profile tab, and this is the most useful distinction in the whole app.

### To change a capture setting

ISO, white balance, clipping, dynamic range, format, resolution.

- Load the profile you want to change.

- Change the setting on the shooting screen, the normal way. Not inside Edit Profile.

- Tap the profile tab.

- Tap Save Changes.

- Open Edit Profile to confirm the new value stuck.

If Save Changes is missing That just means the profile already matches your current settings. The button only appears when there is something to save.

Why an unsaved change disappears This is very likely what happened at Hyannis. You raised the ISO to 400 mid-shoot because the shutter was too slow, but did not save it, and some files came back at ISO 80. Auto-save is deliberately off, so any change you make in the field lives only until the profile is loaded again. Then the saved value quietly takes over. Both profiles are now saved at 400, which removes the problem: your starting point is high enough that any adjustment you make will be downward, and a lost adjustment leaves you no worse off than where you began.

### Edit Profile is for the rest

Auto-save, storage location, and viewfinder items live inside Edit Profile, which is also where you look to confirm what a profile currently holds. Close it with the X in the black panel at the top.

- Auto-save stays off, per profile, so field tweaks never quietly rewrite your saved starting points.

- Photo and video profiles are separate by design. Settings do not carry across.

- Profiles are not available while the phone is locked. Unlock first, then load.

## Where your shots are being saved   UPDATED 28 AUG

This one caused real trouble on the Hyannis shoot, and it turned out not to be operator error at all.

### What was happening

Your profile was set to the photo library. Partway into shooting, the little icon changed to the card symbol and images were going to the SD card instead. You had not touched anything.

The cause: a setting called Auto Connect, under the hardware icon. It tells the app to switch to external storage automatically whenever an external device appears. So the moment the phone slid into the grip with a card in it, Moment quietly redirected everything to the card.

### The fix, done in the session

- Tap the hardware icon, the small hexagon.

- Tap Location.

- Under External Storage, turn Auto Connect off.

- Set the location back to Photo Library.

- Tested by removing the phone from the grip and reinserting it. The location stayed on Photo Library.

Choosing the card on purpose With Auto Connect off, nothing stops you using the card, it just becomes a deliberate choice. Go to the hardware icon, then Location, and select the card. Video especially is often better off there given the file sizes. The card symbol on the shooting screen is your confirmation. It is a small, easy thing to miss, which is exactly why it went unnoticed for most of a shoot. A glance at it belongs on the checklist now.

# Part 3.  In the field

## Exposure: ISO priority for photo   UPDATED 28 AUG

The reason for this mode. With an aperture this wide, outdoor shutter speeds land in the thousands, where choosing between 1/4000 and 1/8000 means nothing. So set the ISO and let the app calculate the shutter.

|  | The setting |
| --- | --- |
| Mode | ISO, in both Natural and ProRAW. |
| Starting ISO | 400, saved into both profiles. ● |
| Why 400 rather than lower | Your own reasoning from the Hyannis shoot, and it is sound: coming down from a higher ISO is easy, while discovering mid-shoot that the shutter is too slow to hand hold costs you frames. With the ND on, a low ISO drops the shutter further than you want. ● |
| Where the shutter reads | Just above the shutter button, labelled SS. In ISO mode that number is feedback rather than a control. |
| Your exposure lever | EV, plus the ND outdoors. If the frame is hot, nudge EV down or dial the ND, and let the shutter fall where it falls. |

The one time shutter mode comes back for photo Deliberate motion blur. To let a crowd smear, switch to shutter mode, drag down to something like 1/15, and use the ND to hold the exposure. That is the one case where the shutter number is a creative choice, and it is a lovely effect worth playing with.

## Focus   UPDATED 28 AUG

Worth trusting the camera here. On the Hyannis shoot you stopped tapping the white box and let the camera choose, and everything came back sharp.

- The lens is wide and the aperture is fixed, which gives deep depth of field. Most of the frame is in focus whether you intervene or not.

- Tapping means taking a hand off the grip, and by the time you have tapped, the subject has usually moved.

- The exception is close work. If you are near your subject, depth of field shrinks and it is worth tapping to place focus deliberately.

## Reading the histogram, waveform, and zebras

Tap the histogram to switch to the waveform. They answer different questions.

|  | Histogram | Waveform |
| --- | --- | --- |
| What it tells you | How much of the image is blown out. | Which part of the frame is blown out. Left of the display is left of your frame. |
| The warning sign | A spike against the right edge. | A solid white line along the top. |
| Looking good | Distribution inside the frame, not pinned to either edge. | Mostly in the middle band. |

- Zebras live in Quick Actions under Clipping, in both photo and video. Press and hold anywhere on the Clipping square to change the threshold: Highlights at 99%, shadows off.

- At 99% the zebras only fire at the true limit. A few sparkles on water or a specular highlight are fine to leave. Zebras spreading across a face or a sky you want texture in are the cue to pull back.

- Both displays are small and unlabelled. That is the app, not you.

## The variable ND filter

The aperture is fixed, so the ND is your only way to cut light without touching shutter or ISO. Outdoors it goes on for stills as well as video.

- For video at 1/48, the ND does the exposure work. Dial it until the zebras clear.

- For photo in ISO mode, it keeps the whole system in a comfortable range on bright days, and it is what makes slow-shutter motion blur possible in daylight.

Keep away from the darkest extreme Your test confirmed it: cranked to maximum and panned across the horizon, artifacts appear. It did not take much backing off for them to clear, so the working range is everything short of the last stretch of the ring. At moderate density the artifact is subtler on the monitor, so when you have been near the extreme, ease off a touch more than looks necessary.

## Contrasty light: ProRAW first

Confirmed side by side: pointed at a bright sky, ProRAW holds the highlights noticeably better than Natural, because Fusion is stacking exposures while Natural works from a single frame.

| Situation | The move |
| --- | --- |
| Contrasty light, indoor or outdoor | ProRAW with HDR. The default for high contrast, not just for shots headed to print. |
| Natural, when you want that look | Ride EV at -0.3 to -0.7. The sensor holds shadows far better than highlights, and the raw file is there for lifting them later. |
| Video | 4K profile, ND dialed in, watch the zebras. |

Still unsettled: Light versus Strong Natural’s dynamic range control behaved differently on the two phones: Strong reduced clipping on one and changed nothing visible on the other. Light remains the setting for now, and this is worth a more controlled look sometime.

## Blocking interruptions

### Messages

Swipe down from the top right corner, tap the moon labelled Focus. The moon by the clock confirms it, and reminds you to turn it off after. Messages still arrive, they just stop dropping banners over your viewfinder.

### The camera roll stuck in vertical

If the camera roll views only in portrait while the viewfinder rotates fine, that is the iPhone rotation lock. Moment’s viewfinder overrides it, the roll does not. Swipe down from the top right and tap the red padlock icon to unlock rotation.

### Sharing several images at once

Moment’s camera roll is a window onto the same photo library, and it does not allow selecting several images to AirDrop or send. For that, switch to the Photos app, select, and share from there.

## When something goes wrong

|  | How | Use when |
| --- | --- | --- |
| Force quit the app | Swipe up from the bottom about a third of the way and hold, then swipe the app card up and away. | First thing to try. Fast and harmless. |
| Restart the phone | Swipe down from the top right, press and hold the power symbol, slide to power off. Or hold the side button and either volume button until the slider appears. | An app is stuck and force quitting did not clear it. This is what resolved the stuck-recording episode. |
| Force restart | Press and release volume up, press and release volume down, then hold the side button until the Apple logo appears. | The phone itself is frozen. |

# Part 4.  After the shoot: the media workflow

## The shape of it

One session folder holds the whole shoot Everything from a shoot day, photo and video, from the camera roll and from the SD card, lands inside that day’s Capture One session folder on the primary drive. The session is where all the work happens, and it is what gets backed up. The archive drive comes later, and separately: a plain copy of the raw captures plus the finished, copyrighted files, in folders anyone could navigate without Capture One.

## Before the shoot: set up the session

- In Capture One, create a new session on the iPhone Capture One Sessions Primary drive, named with the date and event: 20260813_EVENT_SESSION.

- Rename the session subfolders with the same full prefix: the capture folder, Selects, Output, and Trash.

- In Finder, inside the capture folder, make two subfolders, also carrying the full name: one for the camera roll, one for the SD card.

- The full date and event prefix on every folder means any folder that gets moved or copied still says exactly what it is.

- Doing this before the shoot means the folders are waiting for you at the end of a long day.

## End of day, step by step

- On the laptop, open Photos and let the day finish syncing from iCloud. Plug the laptop in if syncing pauses. Then select the day’s shots and use File, Export, Export Unmodified Originals, into the camera roll capture subfolder.

- Copy the SD card’s contents into the SD card subfolder. Everything together, photos and video, then compare sizes with the card to confirm the copy is complete.

- In Capture One, batch rename everything, video included: control click, Batch Rename, find and replace the prefix with the date and event, 20260813_EVENT_.

- Copy/drag (no Chronosync) the whole session folder to the backup drive. Your originals now exist in two places.

- Now it is safe to clear the phone: delete the day’s images in Photos on the laptop, which also removes them from iCloud and the phone in one move.

- Edit in Capture One as usual: rate, tone, and export with your recipe into the session Output folder.

- Run the copyright pass in Photo Mechanic, then the rename. The next section covers this.  ●

- Copy the raw captures and the finished copyrighted folder to the archive drive, under a parent folder named for the date and event.  ●

- Run the ChronoSync task, iPhone Capture One Sessions Primary to Backup, so your edits and exports land on the backup drive too.

One small check during the rename The prefix to replace can vary. Photos come in as IMG. Video names use the long timecode format regardless of your settings, so glance at what the clips are actually called and set the find-and-replace to match.

The order matters in one place Two copies before anything gets deleted. Everything else in the sequence has some give, but step 4 always comes before step 5.

## Copyright, before anything goes out   UPDATED 28 AUG

Capture One does not reliably embed your copyright on export. It can look as though it is there when it is not. Photo Mechanic does it properly, so it becomes the last step before any file leaves your hands.

### Part one: embed the copyright

- In Capture One, choose your recipe: JPEG, 2500 px. Check that the four boxes at the top of the Capture One window are orange.

- Export the images into the session Output folder.

- Open Photo Mechanic.

- In Capture One, go to Output and find the folders for the camera roll and the SD card.

- Drag one folder at a time onto the Photo Mechanic icon in the dock.

- Command A to select all.

- Command I to open the IPTC window. It is also under Image in the top menu.

- Click Load to find your copyright template. In Desktop Working Files, open PHOTO MECHANIC TEMPLATE from DEK, then in the next column choose MCS2026_IPTC.XMP and click Open.

- Scroll down to Image Rights and confirm it looks right.

- Click Apply Template to Selected.

### Part two: verify it took

Bring one image into Photoshop, then File, File Info. The metadata appears there. If a field looks cut off in the display, select all and copy it into a note to see the full text. That truncation is a Photoshop display quirk, not missing data.

### Part three: tag the filenames

So that a copyrighted file announces itself even after it has been moved or pulled into a group of selects.

- Still in the session Output folder, in Finder, select all the files you just processed.

- Right click any one of them and choose Rename.

- In the format dropdown at the top left, change it from Format to Add Text.

- Type your tag, with a leading underscore: _C2026. Leave the position set to after the name, which is the default.

- Click Rename. Command Z undoes it if something looks wrong.

- Rename the enclosing folder by hand to note the copyright as well.

Why the rename happens here and not earlier The tag should only ever appear on files that genuinely carry the copyright, so it goes on after Photo Mechanic has written the metadata, never before. Doing it in Capture One at rename time would put the tag on files that had not been through the copyright step yet. And doing it while the files are still in the session means they arrive on the archive drive already named and already carrying their metadata. Nothing to redo on the other side.

The plain-text symbol C works better than the © character, which can occasionally cause trouble in filenames.

## Your drives

| Drive | Role |
| --- | --- |
| iPhone Capture One Sessions Primary | The working drive, and where all the real work lives. One session per shoot, holding originals, edits, and outputs (including final TIFF). |
| iPhone Capture One Sessions Backup from Primary | The mirror, kept current by the ChronoSync task. Because the sessions hold the edits, this is the backup that matters most. |
| iPhone Raw Captures Archive Primary | The long view. Raw captures plus finished copyrighted files, in plain folders. Also final TIFF file. See below. |

### The ChronoSync task

- Named iPhone Capture One Sessions Primary to Backup. Left side: the primary drive with Admin Access. Right side: the backup drive with Admin Access. Backup left to right. Synchronize deletions; archive deletions.

- Run it after an edit session so ratings, toning, and exports flow across. The first copy of a fresh session can be a simple Finder drag.

- If it ever throws errors on individual files, check whether the target drive is full before suspecting anything worse. ChronoSync’s own archive of old versions is usually what fills it, and from the top menu choose Actions -> Archive -> Clean Up and choose the amount archive to delete.

## The archive drive, separately   UPDATED 28 AUG

Archiving stays out of the end of day rush. It can happen at the end of a project, or whenever it suits.

### The structure, confirmed in the session

- A parent folder named for the date and event.

- Inside it, the raw captures, with the camera roll and SD card folders as they came off the session. Video included.

- Alongside those, the finished copyrighted output folder.

- Also includes the final TIFF, if it exists.

What the archive is for The sessions drive holds the work. The archive holds something different: files a person could open in twenty years without owning Capture One. That is also the argument for eventually putting a large TIFF of your finished images there, since a TIFF will still open long after any particular application has come and gone. It also means your original captures exist in three places once a shoot is archived: the session, the session backup, and here.

- Exhibition prints and finals, the ones Bob Korn makes, belong on the archive drive in their own area, apart from working sessions.

- Copyright registration for finals, and how the Copyright Office’s naming rules link back to your own names, is still on the list to work out.

# Part 5.  Reference: set once, then forget

Worth a spot check after a major Moment update, and otherwise fine to leave alone.

## Moment settings   UPDATED 28 AUG

Gear icon, lower left of the shooting screen.

| Setting | Set to | Why |
| --- | --- | --- |
| File Name Format | Default ● | Gives the familiar incrementing IMG numbers. The Date option produces a long timecode string instead, which is what you had been fighting. Tested both ways in the session. Numbers skip when shooting RAW plus JPEG, two files per frame, which is normal. ● |
| Combine RAW and Processed | Off | Keeps the raw and JPEG as separate files. |
| Preserve Settings | On | Mostly moot, since a loaded profile wins anyway. Harmless. |
| Return to Camera on Launch | On | Opens straight into the viewfinder. |
| Ignore System Portrait Lock | On | The viewfinder rotates even with rotation lock on. The camera roll does not, which is a phone setting, covered in Part 3. |
| Lock Auto White Balance on Record | On | No colour drift mid-clip when in Auto. |
| Show Record Border | On | The red border confirms recording. |
| Shutter Sound | On | Audible confirmation. |
| Left-Handed Interface | Leave as is | No visible difference when toggled. |
| Apply De-Squeeze When Saving | Ignore | Anamorphic lenses only. |
| Auto-save (per profile, in Edit Profile) | Off | Keeps field tweaks from rewriting your profiles. |
| Auto Connect (hardware icon, Location) | Off ● | The one that was switching your storage to the SD card on its own whenever the phone went into the grip. ● |

Video ignores the file name setting Whatever you choose, video clips come out with the long timecode name. Nothing to fix, just something to know when you are setting up the batch rename in Capture One.

Two things you can stop wondering about The ProRAW section in iPhone Settings > Camera > Formats only affects Apple’s stock camera. Tested directly: toggling it changes nothing in Moment. Photo Quality (Speed, Balanced, Quality) was a control in the previous version of the app. In Pro Camera II it is folded into Fusion and the dynamic range enhancer, which are already set.

## iPhone settings

Settings, then Camera. Moment overrides most of these while running; they matter for grab shots in the stock camera.

| Setting | Value |
| --- | --- |
| Record Video | 4K at 24 fps |
| Record Slow-Mo | 4K at 120 fps |
| Camera Capture format | High Efficiency, HEIC and HEVC |
| Photo Mode | 24 MP |
| ProRAW and Apple ProRes | Off. ProRAW happens through Moment. |
| Camera Control button | Double-click opens Moment |
| Visual Intelligence | Off |
| Require Screen On | On, so the camera does not launch in your pocket |
| Photographic Styles | Standard |
| Portraits in Photo Mode | Off |
| Prioritize Faster Shooting | On |
| Lens Correction and Lens Cleaning Hints | On |
| Default Lens | 24mm. The 1x button cycles 24, 28, 35. |
| Grid and Level | Off in the stock app; Moment’s own are on. |
| Preserved Settings | Off |
| Rotation lock (Control Center) | Off when reviewing in the camera roll, since the roll obeys it. |

## Why these formats, in plain terms

Background only. Here for the moment you wonder why a choice was made.

| Format | What it actually is | What that means for you |
| --- | --- | --- |
| Natural | Moment’s own tone mapping on the raw sensor data, without Apple’s sharpening or noise reduction. | The most film-like, closest to a straight capture. A single exposure, so it clips first, which is why EV rides a little low. |
| ProRAW | A 16-bit DNG blending raw sensor data with Apple’s computational stacking. | The best editing latitude and highlight handling on the phone, and the only route to 48 MP. Big files. |
| H.264 video | The older codec, 4:2:0, SDR only. | Less grading room than HEVC, and it edits in Premiere without proxies, which is the trade you chose. The Sony a7 III shipped with it for years. |

### HDR and SDR, stated plainly

HDR looks right on your iPhone and can read differently on someone else’s screen. SDR is safer for handoff. But 48 MP requires HDR, and resolution is the reason to reach for ProRAW, so ProRAW stays HDR and Natural, where most shooting happens, stays SDR.

## Open questions and next sessions   UPDATED 28 AUG

### Next session

- Video into Premiere Pro: import, build a sequence, export something small enough to post. Two routes worth seeing: the full Premiere path, and Media Encoder on its own for when a single clip just needs to be made smaller, which skips Premiere entirely.

### Queued

- A Capture One catalog above your sessions, so everything becomes searchable in one place. Written up separately.  ●

- A second monitor for the 13 inch laptop. Two directions: the Apple Studio Display at around $1,300, where the 5K resolution makes text noticeably crisper, or an inexpensive 4K panel around $200 to $300, lighter and easier to travel with and no great loss if it gets knocked about.  ●

- A low light profile.

- Voice captions in the field: wearing the Zoom F2 and speaking names and details as you shoot, then letting transcription sort it afterward.

- Blurring faces for privacy in photo and video.

- A Sony refresher, before the a7 gathers any more dust.

### Watching

- The shutter reading discrepancy between the two phones at matched settings.

- Natural at Light versus Strong dynamic range, which behaved differently per phone.

- The stuck-recording bug, if it ever recurs.

Moment Field Guide, Version 3, 28 August 2026. Compiled from the working sessions of 25 May, 25 July, 30 July, 6 August, 13 August, and 28 August 2026. Where a question is still open, the guide says so.